# Initial Fire Engineer / Pump Operations

Quiz — 10 Questions

Prepared by Mileage Design | April 2026

Name: ________________________________________

Date: ____________________

Instructions: Circle the best answer for each question.

1. Which NFPA standard governs Fire Apparatus Driver/Operator Professional Qualifications?

A) NFPA 1001

B) NFPA 1041

C) NFPA 1002

D) NFPA 1500

2. "Due regard" means the emergency vehicle driver must:

A) Always have right of way under lights and siren

B) Operate with the safety of all persons in mind regardless of warning devices

C) Drive at any speed necessary to reach the call

D) Never stop at red lights when responding

3. A Storz coupling connects to a hydrant by:

A) A quarter turn clockwise — sexless coupling, no threads

B) Five full clockwise turns with a hydrant wrench

C) Threading the male end into the female outlet

D) A friction-fit clamp tightened with a strap wrench

4. In a forward lay, the engine:

A) Drops a firefighter at the hydrant, then drives to the fire laying supply line behind it

B) Arrives at the fire first, then drives back to the hydrant

C) Pumps from tank water only, never connecting to a hydrant

D) Stages two blocks away and waits for command

5. A typical 1.75-inch handline is supplied at what nozzle pressure (smooth-bore not used)?

A) 50 PSI

B) 75 PSI

C) 100 PSI

D) 150 PSI

6. When backing the apparatus, the spotter should be positioned:

A) At the front bumper

B) At the left rear of the apparatus, in the driver's mirror sight line

C) Inside the cab giving verbal directions

D) Wherever convenient — backing alone is acceptable in an empty bay

7. The correct sequence for engaging a midship pump is:

A) Drive → Pump shift → Park brake → Tank-to-pump

B) Pump shift → Drive → Park brake → Tank-to-pump

C) Park brake → Neutral → Pump shift → Drive → Tank-to-pump

D) Tank-to-pump → Pump shift → Drive → Park brake

8. When positioning the engine at a residential structure fire, you should:

A) Stop directly in front of the door for shortest hose stretch

B) Park on the lawn for stability

C) Pull into the driveway and block the truck company

D) Pull approximately 25 feet past the structure to leave room for the truck company

9. Which of the following is NOT a typical secondary duty of the engineer once water is flowing?

A) Throwing ground ladders for crew bailout

B) Conducting interior primary search

C) Staging spare SCBA bottles at rehab

D) Pulling a backup 1.75-inch line to the front door

10. The four attributes of fire behavior the pump operator should report to command are:

A) Heat, light, smoke, sound

B) Color, smell, texture, temperature

C) Volume, velocity, density, color

D) Speed, height, angle, distance

# Answer Key

Initial Fire Engineer / Pump Operations — Quiz Answer Key

1. C) NFPA 1002

2. B) Operate with the safety of all persons in mind regardless of warning devices

3. A) A quarter turn clockwise — sexless coupling, no threads

4. A) Drops a firefighter at the hydrant, then drives to the fire laying supply line behind it

5. C) 100 PSI

6. B) At the left rear of the apparatus, in the driver's mirror sight line

7. C) Park brake → Neutral → Pump shift → Drive → Tank-to-pump

8. D) Pull approximately 25 feet past the structure to leave room for the truck company

9. B) Conducting interior primary search

10. C) Volume, velocity, density, color

Confidential — Prepared by Mileage Design